# Writing Robot (2021)

## Preface

This project was from a few years ago, so I’m digging through some blurry linear-algebra memories here. If I ever get more time, I’d like to come back, derive all the math cleanly, and update this page.

Originally, this was supposed to be a hands-on physical robot build with, electronics, and fabrication. Then COVID hit, we ended up working withCoppeliaSim as a simulation environment.

My part of the project was the programming, robot math.

### CoppeliaSim – the simulation environment

We used CoppeliaSim to run the actual 3D simulation. It handled:

- Robot kinematics

- Joint motion

- Pen movement

- Collision checking

- Sensors and environment

### MATLAB – the controller / brain

All the logic lived in MATLAB:

- Path planning

- Inverse kinematics

- Trajectory generation

- Talking to CoppeliaSim

# Background — Quick Robotics Terminology

### Forward Kinematics (FK)

This is the process of taking the robot's known joint variables, like the angles for rotating joints and calculating the exact position and orientation of the end-effector (the pen in this case) in the 3D workspace.

### Inverse Kinematics (IK)

IK is the opposite, given a desired end-effector pose, what joint angles produce that pose?

There are two big types:

- Analytical IK – closed-form math solutions (fast, exact, but only works for special robot geometries)

- Numerical IK – iterative solvers (slower but works for almost anything), this project used this.

Open-source analytical IK generators exist, like:

- https://github.com/rpiRobotics/ik-geo

- https://github.com/uw-biorobotics/IKBT

…but they only work for robots with solvable closed-form IK.

Most modern robotics libraries use numerical IK for general cases (like Modern Robotics Textbook, which most of this is based on).

# Basic Robot Setup

Before doing anything fancy, we had to define the robot mathematically:

- Link lengths (L1, L2, …)

- Screw axes (Slist)

- Home configuration (M matrix)

- Joint limits

- Writing plane transform (board frame)

We also had to define:

- Letter sequence (word to draw)

- Letter spacing

- Local XY point sets for each character

# Trajectory Generation

Here’s the fun part where the robot figures out how to draw letters.

### 1. Load letter shapes

Each letter (A, B, C, O…) is stored as a small set of XY points.

### 2. Transform into the writing board frame

We use a T matrix to convert local letter points → positions on the global writing surface.

### 3. Interpolate the trajectory

Break the letter curves into very small steps (the smaller the step, the smoother the robot draws).

# Inverse Kinematics Pipeline

For each tiny step of the trajectory:

- Use IKinSpace or IKinBody to solve θ

- Use the previous solution as the next initial guess

- If it fails → add small random noise to the guess and retry

# Final Output